Titled

Geostatistical Modeling of NO2 Concentrations and Its Impact on Acute Repiratory Infections (ARI) Prevalence in Mid-2025 in Semarang City

Research Background

Air pollution in Indonesia is mainly driven by industry and transportation. In Semarang, the Air Quality Index (AQI) reached 147, categorized as Unhealthy, with 154,883 Acute Respiratory Infection (ARI) cases reported by mid-2025. Nitrogen dioxide (NO2), a carcinogenic gas, is assumed to be one of the major contributors to air pollution and is suspected to be linked to the high number of ARI cases. Therefore, this study aims to determine the influence of NO2 concentration on the high number of ARI cases, given NO2's carcinogenic properties and its role as a major air pollutant. However, monitoring this issue remains challenging, as Semarang has very limited air quality monitoring stations. Sentinel-5P satellite data offers a potential solution, given its wide spatial coverage and daily data availability. Nevertheless, cloud cover, satellite viewing geometry, and the applied QA filter threshold (≥0.75) result in significant data gaps in the imagery, which need to be addressed before the data can be reliably used for further analysis.

Objectives

- Evaluate ML model performance (RMSE, MAE, R²) in reconstructing missing daily NO2 VCD values from Sentinel-5P imagery - Assess correlation between Sentinel-5P NO2 VCD and ground-based NO2 measurements (DLHK stations), Jan–Jun 2025 - Analyze spatiotemporal distribution and NO2 VCD hotspots in Semarang City, Jan–Jun 2025 - Examine the relationship between NO2 VCD levels and ARI prevalence in Semarang City, Jan–Jun 2025

Tools

- Python - R - GoogleEarthEngine - QGIS

Data

- NO2 Sentinel-5P - NO2 Ground-based station - Meteorology - Elevation - Land Cover - ARI Cases - Population Density - Road Density - NO2 CAMS Global Reanalysis

Method

- Gap-filling NO2: A total of 181 daily Sentinel-5P scenes (January–June 2025) were processed using hierarchical imputation (interpolation, Spatial Pattern Scaling, and CAMS data), followed by a comparison of three models: RF (V1), RF+Lag (V2), and RF-STK (V3). All models evaluated using Group-10-Fold Cross-Validation, metrics for evaluated used R², RMSE, MAE, and RPE. - Validation: BLH-normalized satellite NO2 VCD was validated against ground-based NO2 measurements from the DLHK Mijen station using Spearman correlation and multiple linear regression. - Hotspot Analysis: NO2 hotspots were identified using the top 20% percentile method and validated against road density and industrial point count data. - NO2–ARI Relationship: NO2 VCD from the best model was aggregated at the sub-district level and analyzed using an OLS baseline model, which revealed significant spatial autocorrelation (Moran's I = 0.772); this was subsequently addressed using a GTWR model.

Result & Analysis

- Model Evaluation [Figure] [Table] - RF-STK (V3) is the best model with highest accuracy and most stable on GroupKFold: R² = 0.80 ± 0.06 - Top predictors: NO2 lag-1, wind direction, solar radiation - Kriging residuals show strong spatial structure (range ≈ 8 km), it confirms RF-STK effectively corrects spatial bias - Reconstruction Result [Figure] - On days with >50% valid pixels: Model V1 produced coarse reconstructions that failed to capture TROPOMI's spatial patterns, while Model V2 performed better, generating more consistent patterns due to the lag day-1 feature. Model V3 achieved the best results, producing smoother and more spatially coherent maps through kriging correction. - On days with <50% valid pixels: All three models became less stable, with predictions increasingly dominated by predictor variables rather than actual satellite observations. This suggests that model reliability declines significantly when valid data availability is very limited. - Hotspot Analysis [Figure Hotspot] - Hotspot shift is assumed to be linked to national holidays (Vesak Day, Ascension Day, Eid al-Adha), during which traffic in residential/educational areas like Tembalang dropped sharply and hotspots disappeared - Industrial emissions in the west/north remained high, as these facilities operate continuously regardless of holidays - Road density in hotspot areas was consistently higher than non-hotspot areas, peaking at 12.39 km/km² in June - Industries within hotspot zones rose sharply in May–June (33–34 facilities, ~40% of all industries) - NO2-ARI Relationship [Figure Zonal Statistic] GTWR Model Performance - GTWR performed far better than OLS, with R² rising from 0.031 to 0.681 - Residuals became more normal and stable, and spatial autocorrelation dropped significantly (Moran's I: 0.772 → 0.467) NO2's Influence on ARI - Overall, higher NO2 levels were associated with higher ARI prevalence (positive median coefficient) - This relationship was strongest and most consistent in East Semarang, likely due to heavy traffic, industrial activity, and dense population - In other areas, the effect appeared only occasionally rather than continuously - Importantly, every sub-district showed a positive relationship, though the strength differed from place to place - NO2 and ARI cases tended to peak together around January and late February–March, coinciding with the dry season when pollutants stay trapped near the ground

Titled

Geostatistical Modeling of NO2 Concentrations and Its Impact on Acute Repiratory Infections (ARI) Prevalence in Mid-2025 in Semarang City

Research Background

Air pollution in Indonesia is mainly driven by industry and transportation. In Semarang, the Air Quality Index (AQI) reached 147, categorized as Unhealthy, with 154,883 Acute Respiratory Infection (ARI) cases reported by mid-2025. Nitrogen dioxide (NO2), a carcinogenic gas, is assumed to be one of the major contributors to air pollution and is suspected to be linked to the high number of ARI cases. Therefore, this study aims to determine the influence of NO2 concentration on the high number of ARI cases, given NO2's carcinogenic properties and its role as a major air pollutant. However, monitoring this issue remains challenging, as Semarang has very limited air quality monitoring stations. Sentinel-5P satellite data offers a potential solution, given its wide spatial coverage and daily data availability. Nevertheless, cloud cover, satellite viewing geometry, and the applied QA filter threshold (≥0.75) result in significant data gaps in the imagery, which need to be addressed before the data can be reliably used for further analysis.

Objectives

- Evaluate ML model performance (RMSE, MAE, R²) in reconstructing missing daily NO2 VCD values from Sentinel-5P imagery - Assess correlation between Sentinel-5P NO2 VCD and ground-based NO2 measurements (DLHK stations), Jan–Jun 2025 - Analyze spatiotemporal distribution and NO2 VCD hotspots in Semarang City, Jan–Jun 2025 - Examine the relationship between NO2 VCD levels and ARI prevalence in Semarang City, Jan–Jun 2025

Tools

- QGIS

Data

- BPS Semarang - MAPID

Method

Result & Analysis

Conclusion